SECTION 01120

MEASUREMENT AND PAYMENT

PART 1	GENERAL

- REQUIREMENTS INCLUDED AND DEFINITIONS

  - The term “all costs”, as used in the payment descriptions within Part 2, is defined as full compensation for all equipment, labor, material and incidental costs.

  - The Work of this Contract (and subsequent payment) consists of furnishing all equipment, labor, material and incidentals, as well as performing all construction, installation and testing of all improvements, modifications and additions, all as shown on the drawings and detailed in the specifications.

  - All work shown on the drawings or detailed in the specifications and not specifically set forth in the Itemized Proposal (Bid Schedule) as a pay item shall be considered a subsidiary obligation of the contractor, and all costs in connection therewith shall be included in the prices named in the proposal.

  - Progress measurements (for progress payments on the pay applications) shall be determined by the amount of work performed during a given period.

    - Payments for items with a lump sum unit shall be based on one of the three options below, with the payment method being agreed upon by all parties.

      - Field measured to determine the actual value.

      - An estimated value of the work performed.

      - Pro-rated over the life of the contract, based off contract time or based off the total value of work performed percentage.  Example: Construction Field Office.

    - Payments for items with a unit that may be measured to the tenth, may be paid to the tenth. However, rounding will only occur during the final pay application and shall not be applied to any progress measurements.

  - Final measurement is to be applied to the final pay application.  Items not requiring final measurement will pay the full amount on the Bid Schedule, unless appreciable errors are found in the bid quantity, or changes are authorized by the Owner.

  - Rounding shall be performed during the final pay application, and rounding shall be to the nearest whole number, with 0 thru 4 being rounded down, and 5 thru 9 being rounded up.  The following items will not be rounded, and shall be paid to the nearest tenth:

    - Seed and temporary surfacing.

PART 2	BID ITEMS (IN ALPHABETICAL ORDER)

- ADA DETECTABLE WARNINGS

  - Final measurement will not be made unless changes to the bid quantity are authorized.

  - Payment for ADA detectable warnings shall be based on the unit price per square foot as set forth in the Bid Schedule.  Said price shall include all costs necessary to complete the work including, but not limited to, subgrade preparation, forming, concrete base, reinforcing, placing, doweling, jointing, mortar work, saw cutting, finishing and curing, as required by the drawings and specifications.

- AIR RELIEF VALVE ASSEMBLY

  - Final measurement will be based on each completed and installed item.

  - Payment for air relief valve assembly shall be based on the unit price per each as set forth in the Bid Schedule, per size of assembly.  Said price shall include all costs necessary to complete the work as required by the drawings and specifications.

- BORROW MATERIAL (CONTRACTOR FURNISHED)

  - Final measurement will be based on the amount of material used, per the following guidelines:

    - The borrow pit shall be suitably measured (survey cross section) prior to any material being removed and when material from the pit is no longer required.  All survey information shall be provided to Owner.

  - Payment for borrow material shall be based on the unit price per cubic yard as set forth in the Bid Schedule.  The quantity in the bid schedule is the estimated borrow pit volume required to complete the on-site compacted fill areas.  Said price shall include all costs necessary to complete the work including, but not limited to, surveying the site, permitting, site security, segregation of material, erosion control, excavation, loading, hauling placement and compaction, as required by the drawings and specifications.

- CABLE OR WIRING (ALL TYPES)

  - Final measurement will not be made unless changes to the bid quantity are authorized.

  - Payment for cable or wiring shall be based on the unit price per linear foot as set forth in the Bid Schedule, per type or size of cable or wiring.  Said price shall include all costs necessary to complete the work including, but not limited to, connectors, hardware, termination, fuse assemblies and tags, as required by the drawings and specifications.   Fiber Optic cable shall include splicing of fibers in splice cabinets and fiber distribution units, termination and testing of the fiber optic cables to provide a fully functioning system.

- CLEARING AND GRUBBING

  - Final measurement will not be made unless changes to the bid quantity are authorized.

  - Payment for clearing and grubbing shall be based on the lump sum price as set forth in the Bid Schedule.  Said price shall include all costs necessary to complete the work including, but not limited to, demolishing, removing and disposing of all trees, shrubs, bushes, stumps and roots within the construction limits, as required by the drawings and specifications.

- COLD MILLING

  - Final measurement will not be made unless changes to the bid quantity are authorized.

  - Payment for cold milling shall be based on the unit price per square yard as set forth in the Bid Schedule, per depth of milling.  Said price shall include all costs necessary to complete the work including, but not limited to, saw cutting and removal, as required by the drawings and specifications.  Overlay will be paid separately as a pavement bid item.

- CONDUIT (ALL TYPES)

  - Final measurement will not be made unless changes to the bid quantity are authorized.

  - Payment for conduit shall be based on the unit price per linear foot as set forth in the Bid Schedule, per type or size of conduit.  Said price shall include all costs necessary to complete the work including, but not limited to, proper jointing, labeling and water proofing, as required by the drawings and specifications.

- CONSTRUCTION OFFICE

  - Final measurement will not be made unless changes to the bid quantity are authorized.

  - Payment for the construction office shall be based on the lump sum price as set forth in the Bid Schedule.  Said price shall include all costs for providing specified furnishings, utility connection and usage changes (electric, heat and phone) and other incidentals required to maintain the office for the duration of the project.  The appearance and condition of the construction office is subject to the approval of the Owner prior to payment.

- CURB AND GUTTER (ALL TYPES)

  - Final measurement will not be made unless changes to the bid quantity are authorized.

  - Payment for curb and gutter shall be based on the unit price per linear foot as set forth in the Bid Schedule, per the type of curb and gutter.  Said price shall include all costs necessary to complete the work including, but not limited to, subgrade preparation, forming, reinforcing, placing, doweling, jointing, deflector construction, weep holes, finishing, curing and backfilling, as required by the drawings and specifications.

- DETECTORS (TRAFFIC SIGNALS)

  - Final measurement will be based on each completed and installed item.

  - Payment for push button detectors, microwave detectors, induction probe detectors, and detector cards shall be based on the unit price per each as set forth in the Bid Schedule, per type of detector.  Said price shall include all costs necessary to complete the work including, but not limited to, card rack assembly and card rack power supplies, as required by the drawings and specifications.

- DEMOLITION AND REMOVAL

  - Final measurement will not be made unless changes to the bid quantity are authorized.

  - Payment for demolition and removal shall be based on the lump sum price as set forth in the Bid Schedule.  Said price shall include all costs necessary to complete the work including, but not limited to, demolishing, removing, disposing, plugging and capping of all structures and improvements, such as existing pavement, curb, sidewalk, pipes and structures (such as sanitary, storm and water) fences, poles and footings, within the construction limits, unless included in other items of work, as required by the drawings and specifications.

- DRAIN TILE (EDGE DRAIN OR UNDER DRAIN)

  - Final measurement will not be made unless changes to the bid quantity are authorized.

  - Payment for drain tile shall be based on the unit price per linear foot as set forth in the Bid Schedule, per type or size of the installed item.  Said price shall include all costs necessary to complete the work including, but not limited to, excavation, filter fabric, wire mesh, fittings and gravel, as required by the drawings and specifications.  Drain tile installed in retaining walls will be considered subsidiary to the retaining wall pay item.

- DRAINABLE BASE

  - Final measurement will not be made unless changes to the bid quantity are authorized.

  - Payment for drainable base shall be based on the unit price per square yard as set forth in the Bid Schedule, per type of material and thickness of the installed item.  Said price shall include all costs necessary to complete the work including, but not limited to, placing, compacting, rolling and watering as required by the drawings and specifications.

- DRIVEWAYS OR ENTRANCES (PAVEMENT OR GRAVEL)

  - Final measurement will be based on the square yard of the completed and installed item.

  - Payment for driveways shall be based on the unit price per square yard as set forth in the Bid Schedule, per type of pavement and thickness of the installed item.  Said price shall include all costs necessary to complete the work including, but not limited to, forming, reinforcing, placing, compacting, saw cutting, milling, doweling, jointing, curing and sealing, as required by the drawings and specifications.

- EDGE TREATMENT (TEMPORARY)

  - Final measurement will based on the horizontal length of the completed and installed item.  No location will be measured more than once, regardless of the number of applications.

  - Payment for edge treatment shall be based on the unit price per linear foot as set forth in the Bid Schedule.  Said price shall include all costs necessary to complete the work including, but not limited to, maintenance, removal, replacement and disposal, as required by the drawings and specifications.

- EMBANKMENT

  - Final measurement will not be made unless changes to the bid quantity are authorized.

  - Payment for embankment shall be based on the unit price per cubic yard as set forth in the Bid Schedule.  The quantity in the bid schedule identifies the compacted fill volume computed from the cross sections.  Said price shall include all costs necessary to complete the work including, but not limited to, roadway grading, loading, hauling, placement, compacting, finish grading, subgrade preparation and reconstruction, topsoil replacement (including additional materials to sustain plant growth), as required by the drawings and specifications.

- EMERGENCY VEHICLE DETECTION SYSTEM (TRAFFIC SIGNALS)

  - Final measurement will not be made unless changes to the bid quantity are authorized.

  - Payment for emergency vehicle detection system shall be based on lump sum price per each intersection as set forth in the Bid Schedule.  Said price shall include all costs necessary to complete the work as required by the drawings and specifications.

- ENCASEMENT

  - Final measurement will be based on the horizontal length of the completed and installed item.

  - Payment for encasement shall be based on the unit price per linear foot as set forth in the Bid Schedule, per type of encasement.  Said price shall include all costs necessary to complete the work as required by the drawings and specifications.

- FENCE (ALL TYPES)

  - Final measurement will be based on the horizontal length of the completed and installed item.

  - Payment for fence shall be based on the unit price per linear foot as set forth in the Bid Schedule, per type of fence.  Said price shall include all costs necessary to complete the work including, but not limited to, gates (permanent or temporary), maintenance and removal of temporary fence, as required by the drawings and specifications.

- FIRE HYDRANT ASSEMBLY (WITH NEW HYDRANT)

  - Final measurement will be based on each completed and installed item.

  - Payment for fire hydrant installation shall be based on the unit price per each as set forth in the Bid Schedule.  Said price shall include all costs necessary to complete the work including, but not limited to, all excavation (earth, rock, shale), dewatering, bedding, polyethylene encasement, placing, pipe to pipe connections, restraint measures, flushing, disinfection, pressure testing, backfilling, compacting, grading and removal of excess or unsuitable material, as required by the drawings and specifications.

- FIRE HYDRANT ASSEMBLY RELOCATION (WITH REUSE OF EXISTING HYDRANT)

  - Final measurement will be based on each completed and installed item.

  - Payment for fire hydrant relocation shall be based on the unit price per each as set forth in the Bid Schedule.  Said price shall include all costs necessary to complete the work including, but not limited to, all excavation (earth, rock, shale), dewatering, bedding, polyethylene encasement, placing, pipe to pipe connections, restraint measures, flushing, disinfection, pressure testing, backfilling, compacting, grading and removal of excess or unsuitable material, as required by the drawings and specifications.

- GUARDRAIL

  - Final measurement will based on the horizontal length of the completed and installed item.

  - Payment for guardrail shall be based on the unit price per linear foot as set forth in the Bid Schedule.  Said price shall include all costs necessary to complete the work, as required by the drawings and specifications.

- GUARDRAIL – BRIDGE ANCHOR OR BRIDGE TRANSITION SECTION

  - Final measurement will be based on each completed and installed item.

  - Payment for guardrail (bridge anchor or bridge transition section) shall be based on the unit price per each as set forth in the Bid Schedule.  Said price shall include all costs necessary to complete the work, as required by the drawings and specifications.

- GUARDRAIL END TERMINAL

  - Final measurement will be based on each completed and installed item.

  - Payment for guardrail end terminal shall be based on the unit price per each as set forth in the Bid Schedule.  Said price shall include all costs necessary to complete the work, as required by the drawings and specifications.

- LUMINAIRES

  - Final measurement will be based on each completed and installed item.

  - Payment for luminaires shall be based on the unit price per each as set forth in the Bid Schedule, per type of luminaire.  Said price shall include all costs necessary to complete the work, as required by the drawings and specifications.

- MANHOLE REBUILDING

  - Final measurement will be based on each completed and installed item.

  - Payment for manhole rebuilding shall be based on the unit price per each as set forth in the Bid Schedule, per type or size of manhole.  Said price shall include all costs necessary to complete the work including, but not limited to, excavation, dewatering, forming, jointing, and temporary service requirements, as required by the drawings and specifications.

- MEDIAN NOSE

  - Final measurement will not be made unless changes to the bid quantity are authorized.

  - Payment for median nose shall be based on the unit price per each as set forth in the Bid Schedule.  Said price shall include all costs necessary to complete the work including, but not limited to, subgrade preparation, forming, reinforcing, placing, weep holes, doweling, jointing, finishing and curing, as required by the drawings and specifications.

- MESSAGE BOARDS (VARIABLE)

  - Final measurement will be based on the total number of weeks that a specific quantity of boards are on site (i.e. 1 board for 32 weeks, 2 boards for 25 weeks, etc).

  - Payment for variable message boards shall be based on the unit price per week as set forth in the Bid Schedule, per number of boards.  Said price shall include all costs necessary to complete the work including, but not limited to, mobilization, movement and programming, as required by the drawings and specifications.

- MOBILIZATION

  - Partial payment for the mobilization pay item will be based on the contract lump sum bid price for mobilization and will be made in accordance with the following Payment Schedule:

    - Twenty-five percent, when five percent or more of the original contract amount is earned.

    - Fifty percent, when ten percent or more of the original contract amount is earned.

    - Seventy-five percent, when twenty-five percent or more of the original contract amount is earned.

    - One-hundred percent, when fifty percent or more of the original contract amount is earned.

- MODEMS

  - Final measurement will be based on each completed and installed item.

  - Payment for modems shall be based on the unit price per each as set forth in the Bid Schedule. Said price shall include all costs necessary to complete the work including, but not limited to dial-up modem, connectors and cables, as required by the drawings and specifications for a fully functioning system.

- MULTI USE PATH

  - Final measurement will be based on the square yard of the completed and installed item.

  - Payment for multi use paths shall be based on the unit price per square yard as set forth in the Bid Schedule, per type of path and thickness of the installed item.  Said price shall include all costs necessary to complete the work including, but not limited to, forming, reinforcing, placing, compacting, saw cutting, milling, doweling, jointing, curing and sealing, as required by the drawings and specifications.

- PAVEMENT (ALL TYPES)

  - Final measurement will not be made unless changes to the bid quantity are authorized.

  - Payment for pavement shall be based on the unit price per square yard as set forth in the Bid Schedule, per type of pavement and thickness of the installed item.  Said price shall include all costs necessary to complete the work including, but not limited to, forming, reinforcing, placing, compacting, saw cutting, connections to existing pavement, milling, doweling, jointing, curing and sealing, as required by the drawings and specifications.

- PAVEMENT MARKING (PERMANENT – LUMP SUM)

  - Final measurement will not be made unless changes to the bid quantity are authorized.

  - Payment for pavement markings shall be based on the lump sum price as set forth in the Bid Schedule.  Said price shall include all costs necessary to complete the work including, but not limited to, removal of old markings, as required by the drawings and specifications.

- PAVEMENT MARKING (PERMANENT ARROWS)

  - Final measurement will be based on each completed and installed item.

  - Payment for pavement marking (arrows) shall be based on the unit price per each as set forth in the Bid Schedule, per single head arrow or multi headed arrow.  Said price shall include all costs necessary to complete the work, as required by the drawings and specifications.

- PAVEMENT MARKING (PERMANENT STRIPING)

  - Final measurement will not be made unless changes to the bid quantity are authorized.

  - Payment for pavement marking (striping) shall be based on the unit price per linear foot as set forth in the Bid Schedule, per width and type of striping.  Skip stripe payment shall be based on the linear footage of the applied stripe; gaps will not be measured.  Payment for double yellow centerline shall be based on the linear footage of each stripe.  Said price shall include all costs necessary to complete the work, as required by the drawings and specifications.

- PAVERS

  - Final measurement will be based on the square foot of the completed and installed item.

  - Payment for pavers shall be based on the unit price per square foot as set forth in the Bid Schedule.  Said price shall include all costs necessary to complete the work including, but not limited to, all excavation (earth, rock, shale), backfilling, compacting, grading, forming, concrete base or leveling course, reinforcing, jointing, sealing, doweling, sawing, filter fabric and weep holes, as required by the drawings and specifications.

- POSTS (STREET LIGHTS OR TRAFFIC SIGNALS)

  - Final measurement will be based on each completed and installed item.

  - Payment for posts shall be based on the unit price per each as set forth in the Bid Schedule, per type or size of post.  Said price shall include all costs necessary to complete the work as required by the drawings and specifications. Mast arms and luminaire bracket arms are subsidiary to posts.

- POST BASES (CONCRETE BASES FOR STREET LIGHTS OR TRAFFIC SIGNALS)

  - Final measurement of post bases (concrete) will not be made unless changes to the bid quantity are authorized.

  - Payment for post bases (concrete) shall be based on the unit price per 1/10 cubic yard as set forth in the Bid Schedule, per type or size of post base (concrete).  Said price shall include all costs necessary to complete the work including, but not limited to, excavation, removal, and disposal encountered within the construction limits, ground rods, ground wire, and steel, as required by the drawings and specifications.

- POST BASES (SCREW IN BASES FOR STREET LIGHTS)

  - Final measurement will be based on each completed and installed item.

  - Payment for post bases (screw in) shall be based on the unit price per each as set forth in the Bid Schedule, per type or size of post base (screw in).  Said price shall include all costs necessary to complete the work as required by the drawings and specifications.

- POWER SUPPLY ASSEMBLIES (STREET LIGHTS OR TRAFFIC SIGNALS)

  - Final measurement will be based on each completed and installed item.

  - Payment for power supply assembly shall be based on the unit price per each as set forth in the Bid Schedule, per type of power assembly.  Said price shall include all costs necessary to complete the work including, but not limited to, ground rods, and cable and conduit required for connection, as required by the drawings and specifications.

- PULL BOXES

  - Final measurement will be based on each completed and installed item.

  - Payment for pull boxes shall be based on the unit price per each as set forth in the Bid Schedule, per type or size of pull box.  Said price shall include all costs necessary to complete the work as required by the drawings and specifications.

- RETAINING WALL

  - Final measurement will be based on the exposed face of the wall, per square foot, of the completed and installed item.

  - Payment for retaining wall shall be based on the unit price per square foot as set forth in the Bid Schedule, based on the exposed face of the wall.  Said price shall include all costs necessary to complete the work including, but not limited to, all excavation (earth, rock, shale), backfilling, compacting, grading, forming, dewatering, shoring, concrete base or leveling course, reinforcing, jointing, sealing, doweling, sawing, geogrid, filter fabric, drain tile and weep holes, as required by the drawings and specifications.

- RIP RAP (ALL TYPES)

  - Final measurement will be based on the square yard of the completed and installed item.

  - Payment for rip rap shall be based on the unit price per square yard as set forth in the Bid Schedule, per thickness of rip rap.  Said price shall include all costs necessary to complete the work including, but not limited to, filter fabric, all excavation (earth, rock, shale), backfilling, compacting, grading and removal of excess or unsuitable material, as required by the drawings and specifications.

- SANITARY SEWER CONNECTIONS (MAIN LINES)

  - Final measurement will not be made unless changes to the bid quantity are authorized.

  - Payment for sanitary sewer connections shall be based on the unit price per each as set forth in the Bid Schedule. Said price shall include all costs necessary to complete the work including, but not limited to, all excavation (earth, rock, shale), bedding, placing, pipe to pipe connections, sealing, backfilling, compacting, grading and removal of excess or unsuitable material, as required by the drawings and specifications.

- SANITARY SEWER PIPE (ALL TYPES)

  - Final measurement will not be made unless changes to the bid quantity are authorized.

  - Payment for sanitary sewer pipe shall be based on the unit price per linear foot as set forth in the Bid Schedule, per type or size of pipe. Said price shall include all costs necessary to complete the work including, but not limited to, all excavation (earth, rock, shale), bedding, placing, pipe to pipe connections, sealing, backfilling, compacting, grading and removal of excess or unsuitable material, as required by the drawings and specifications.

- SANITARY SEWER STRUCTURES (MANHOLES)

  - Final measurement will be based on each completed and installed item.

  - Payment for sanitary sewer structures shall be based on the unit price per each as set forth in the Bid Schedule, per type or size of structure. Said price shall include all costs necessary to complete the work including, but not limited to, all excavation (earth, rock, shale), bedding, placing or building structure, invert construction, structure to pipe connections, final grade adjustments to the top, sealing, curing, backfilling, compacting, grading and removal of excess or unsuitable material, as required by the drawings and specifications.

- SANITARY SEWER WYE AND SERVICE CONNECTIONS

  - Final measurement will be based on each completed and installed item.

  - Payment for sanitary sewer wye and service connections shall be based on unit price per each as set forth in the Bid Schedule.  Said price shall include all costs necessary to complete the work including, but not limited to, all excavation (earth, rock, shale), bedding, placing, fittings, concrete cradle, connection of sewer stub and riser, backfilling, compacting, grading and removal of excess or unsuitable material, as required by the drawings and specifications.

- SEDIMENT CONTROL DEVICES (ALL TYPES)

  - Final measurement will be based on counting the number, measuring the linear footage or measuring the area of the completed and installed item.

  - Payment for sediment control shall be based on the unit price as set forth in the Bid Schedule, per linear foot for sediment fence and per each for inlet protection and check dams.  The unit price shall include full compensation for equipment, labor, material, repair, maintenance and incidental costs, including removal of materials.  Removal of sediment, repair and maintenance of installed sediment control shall be subsidiary to sediment control.

- SEEDING (PERMANENT OR TEMPORARY)

  - Final measurement will be based on the acreage of the completed and installed item.

  - Payment for seeding shall be based on the unit price per acre as set forth in the Bid Schedule, per type of seed.  Said price shall include all costs necessary to complete the work including, but not limited to, aeration, fertilization, mulching and watering, as required by the drawings and specifications.  As needed, Contractor shall water seed until final acceptance.

- SIDEWALK

  - Final measurement will be based on the square yard of the completed and installed item.

  - Payment for sidewalks shall be based on the unit price per square yard as set forth in the Bid Schedule, per thickness of the installed item.  Said price shall include all costs necessary to complete the work including, but not limited to, placing, compacting, saw cutting, milling, doweling, jointing, curing and sealing, as required by the drawings and specifications.

- SIDEWALK RAMPS

  - Final measurement will be based on the square yard of the completed and installed item.

  - Payment for sidewalk ramps shall be based on the unit price per square yard as set forth in the Bid Schedule, per thickness of the installed item.  Said price shall include all costs necessary to complete the work including, but not limited to, placing, compacting, saw cutting, milling, doweling, jointing, curing and sealing, as required by the drawings and specifications.

- SOD (ALL TYPES)

  - Final measurement will be based on the square yard of the completed and installed item.

  - Payment for sod shall be based on the unit price per square yard as set forth in the Bid Schedule, per type of sod.  Said price shall include all costs necessary to complete the work including, but not limited to, aeration, fertilization, rolling and watering, as required by the drawings and specifications.  As needed, Contractor shall water sod until final acceptance.

- SPLICE CABINET

  - Final measurement will be based on each completed and installed item.

  - Payment for splice cabinet shall be based on the unit price per each as set forth in the Bid Schedule.  Said price shall include all costs necessary to complete the work including, but not limited to, rack cage, fiber distribution unit, cabinet and grounding as required by the drawings and specifications.

- STORM SEWER END SECTIONS (ALL TYPES)

  - Final measurement will not be made unless changes to the bid quantity are authorized.

  - Payment for storm sewer end sections shall be based on the unit price per each as set forth in the Bid Schedule, per type or size of end section. Said price shall include all costs necessary to complete the work including, but not limited to, all excavation (earth, rock, shale), bedding, placing or building the end section, end section to pipe connections, head and toe wall construction, sealing, curing, backfilling, compacting, grading and removal of excess or unsuitable material, as required by the drawings and specifications.

- STORM SEWER PIPE (ALL TYPES)

  - Final measurement will not be made unless changes to the bid quantity are authorized.

  - Payment for storm sewer pipe shall be based on the unit price per linear foot as set forth in the Bid Schedule, per type or size of pipe. Said price shall include all costs necessary to complete the work including, but not limited to, all excavation (earth, rock, shale), bedding, placing, pipe to pipe connections, sealing, backfilling, compacting, grading and removal of excess or unsuitable material, as required by the drawings and specifications.

- STORM SEWER STRUCTURES (INLETS, JUNCTION BOXES AND MANHOLES)

  - Final measurement will be based on each completed and installed item.

  - Payment for storm sewer structures shall be based on the unit price per each as set forth in the Bid Schedule, per type or size of structure. Said price shall include all costs necessary to complete the work including, but not limited to, all excavation (earth, rock, shale), bedding, placing or building structure, invert construction, structure to pipe connections, weep holes, small subgrade drainage pipes, final grade adjustments to the top, lift hook concealment, sealing, curing, backfilling, compacting, grading and removal of excess or unsuitable material, as required by the drawings and specifications.

- SUBGRADE STABILIZATION (ALL TYPES)

  - Final measurement will not be made unless changes to the bid quantity are authorized.

  - Payment for stabilized subgrade shall be based on the unit price per square yard as set forth in the Bid Schedule, per type of material and thickness of the installed item.  Said price shall include all costs necessary to complete the work including, but not limited to, placing, compacting, rolling and watering as required by the drawings and specifications.

- SYSTEM MASTER FOR TRAFFIC SIGNALS

  - Final measurement will be based on each completed and installed item.

  - Payment for system master shall be based on the unit price per each as set forth in the Bid Schedule.  Said price shall include all costs necessary to complete the work including, but not limited to, master controller, connectors and cables, telephone interface panels, service outlets, door alarm, fiber optic data link, dial-up modem, and all aspects of the system acceptance test, as required by the drawings and specifications.

- SYSTEM SOFTWARE FOR TRAFFIC SIGNALS

  - Final measurement will be based on each furnished item.

  - Payment for system software shall be based on the unit price per each as set forth in the Bid Schedule.  Said price shall include all costs necessary to complete the work as required by the drawings and specifications.  If the city furnishes the system software or has committed to purchase the system software in another contract, no payment will be made for system software.  This shall include versions of previously supplied system software.  Installation and programming will be at the Contractor’s expense.

- TEMPORARY SURFACING (ALL TYPES)

  - Final measurement will be based on the tonnage of the completed and installed item.

  - Payment for temporary surfacing shall be based on the unit price per ton as set forth in the Bid Schedule, per type of the installed item.  Said price shall include all costs necessary to complete the work including, but not limited to, placing, compacting, maintenance and removal, as required by the drawings and specifications.

- TRAFFIC CONTROL (LUMP SUM)

  - Final measurement will not be made unless changes to the bid quantity are authorized.

  - Payment for traffic control shall be based on the lump sum price as set forth in the Bid Schedule.  Said price shall include all costs necessary to complete the work including, but not limited to, temporary striping, arrow boards, construction signs, barricades and channelization devices, as required by the drawings and specifications.  Payment for relocation and reuse of the items shall be considered subsidiary to the payment for the initial installation.

- TRAFFIC CONTROL BARRICADES (ALL TYPES)

  - Final measurement will be based on each completed and installed item.

  - Payment for traffic control barricades shall be based on the unit price per each as set forth in the Bid Schedule, per type of barricade.  Said price shall include all costs necessary to complete the work including, but not limited to, placing, maintaining, cleaning, relocation and reuse, as required by the drawings and specifications.  Traffic control signs mounted on barricades will be paid under the traffic control signing bid item.

- TRAFFIC CONTROL CHANNELIZERS (CONES OR DRUMS)

  - Final measurement will be based on each completed and installed item.

  - Payment for traffic control channelizers shall be based on the unit price per each as set forth in the Bid Schedule, per type of channelizer and the maximum number on the job.  Said price shall include all costs necessary to complete the work including, but not limited to, placing, maintaining, cleaning, relocation and reuse, as required by the drawings and specifications.

- TRAFFIC CONTROL SIGNING

  - Final measurement will be based on the square foot of the completed and installed item.

  - Payment for traffic control signs shall be based on the unit price per square foot as set forth in the Bid Schedule.  Said price shall include all costs necessary to complete the work including, but not limited to, post setting, posts, maintaining, cleaning, relocation and reuse, mounting and breakaway hardware, as required by the drawings and specifications.

- TRAFFIC CONTROL STRIPING

  - Final measurement will be based on the horizontal length of the completed and installed item.

  - Payment for traffic control striping shall be based on the unit price per linear foot as set forth in the Bid Schedule, per width or type of striping.  Skip stripe payment shall be based on the linear footage of the applied stripe; gaps will not be measured.  Payment for double yellow centerline shall be based on the linear footage of each stripe.  Said price shall include all costs necessary to complete the work, as required by the drawings and specifications.

- TRAFFIC CONTROL STRIPING (ARROWS)

  - Final measurement will be based on each completed and installed item.

  - Payment for pavement marking (arrows) shall be based on the unit price per each as set forth in the Bid Schedule, per single head arrow or multi headed arrow.  Said price shall include all costs necessary to complete the work, as required by the drawings and specifications.

- TRAFFIC CONTROL TEMPORARY TRAFFIC SIGNAL

  - Final measurement will not be made unless changes to the bid quantity are authorized.

  - Payment for traffic control temporary traffic signal shall be based on the lump sum price per intersection as set forth in the Bid Schedule.  Said price shall include all costs necessary to complete the work including, but not limited to, poles, guy wires, conduit, cables, heads, controller, power supply, junction boxes, and hardware, as required by the drawings and specifications.  Payment for relocation and reuse of the items shall be considered subsidiary to the payment for the initial installation.

- TRAFFIC CONTROLLER ASSEMBLIES

  - Final measurement will be based on each completed and installed item.

  - Payment for traffic controller assembly shall be based on the unit price per each as set forth in the Bid Schedule.  Said price shall include all costs necessary to complete the work including, but not limited to, controller cabinet, hardware, panels, switches, controllers, flashers, connectors, cables, fiber distribution unit and fiber optic data links (modems), as required by the drawings and specifications.

- TRAFFIC SIGNAL HEADS

  - Final measurement will be based on each completed and installed item.

  - Payment for traffic signal heads shall be based on the unit price per each as set forth in the Bid Schedule, per type or size of head.  Said price shall include all costs necessary to complete the work as required by the drawings and specifications.

- TRAFFIC SIGNS (PERMANENT)

  - Final measurement will be based on the nearest 1/10 square foot for each sign and to the nearest square foot for the total of the completed and installed item.

  - Payment for traffic signs shall be based on the unit price per square foot as set forth in the Bid Schedule.  Said price shall include all costs necessary to complete the work including, but not limited to, post setting, posts, mounting and breakaway hardware, as required by the drawings and specifications.

- TREES (ALL TYPES)

  - Final measurement will be based on each completed and installed item.

  - Payment for trees shall be based on the unit price per each as set forth in the Bid Schedule, per type or size of tree.  Said price shall include all costs necessary to complete the work as required by the drawings and specifications.

- UNCLASSIFIED EXCAVATION

  - Final measurement will not be made unless changes to the bid quantity are authorized.

  - Payment for unclassified excavation shall be based on the unit price per cubic yard as set forth in the Bid Schedule.  Said price shall include all costs necessary to complete the work including, but not limited to, roadway grading, excavation of whatever material is encountered (earth, shale, limestone), loading, hauling, placement, compaction, subgrade preparation and reconstruction, topsoil removal or placement, as required by the drawings and specifications.

- UNSUITABLE MATERIAL REMOVAL & REPLACEMENT

  - Final measurement will be based on survey cross-sections of the area before and after the unsuitable material is removed, or by other method acceptable to the City.

  - Payment for unsuitable material shall be based on the unit price per cubic yard as set forth in the Bid Schedule.  Said price shall include all costs necessary to complete the work including, but not limited to, loading, hauling, and off-site disposal of the unsuitable material at a location provided by the contractor; and the loading, hauling, placing and compacting of suitable on-site replacement material.  Payment for replacement material, other than available on-site earthen materials, will be based on a negotiated unit or lump sum price.

- VIDEO DETECTION SYSTEMS

  - Final measurement will not be made unless changes to the bid quantity are authorized..

  - Payment for video detection system shall be based on the lump sum price per intersection as set forth in the Bid Schedule.  Said price shall include all costs necessary to complete the work as required by the drawings and specifications.

- WASTE (HAUL OFF)

  - Final measurement will not be made unless changes to the bid quantity are authorized.

  - Payment for waste shall be based on the unit price per cubic yard as set forth in the Bid Schedule.  The quantity in the bid schedule identifies the estimated volume of excess cut material to be removed from the site.  Said price shall include all costs necessary to complete the work including, but not limited to, loading, hauling, delivery and placement at a location arranged for by the contractor, as required by the drawings and specifications.

- WATER LINE CONNECTIONS (MAIN LINE CONNECTIONS)

  - Final measurement will be based on each completed and installed item.

  - Payment for water line connections shall be based on the unit price per each as set forth in the Bid Schedule. Said price shall include all costs necessary to complete the work including, but not limited to, all excavation (earth, rock, shale), dewatering, bedding, polyethylene encasement, placing, pipe to pipe connections, restraint measures, flushing, disinfection, pressure testing, backfilling, compacting, grading and removal of excess or unsuitable material, as required by the drawings and specifications.

- WATER LINE FITTINGS (BENDS, CROSSES, REDUCERS, SLEEVES, TEES)

  - Final measurement will be based on each completed and installed item.

  - Payment for water line fittings shall be based on the unit price per each as set forth in the Bid Schedule, per type or size of fitting. Said price shall include all costs necessary to complete the work including, but not limited to, all excavation (earth, rock, shale), dewatering, bedding, polyethylene encasement, placing, pipe to pipe connections, restraint measures, flushing, disinfection, pressure testing, backfilling, compacting, grading and removal of excess or unsuitable material, as required by the drawings and specifications.

- WATER LINE PIPE (ALL TYPES)

  - Final measurement will be based on the horizontal length of the completed and installed item, as measured along the centerline.

  - Payment for water line pipe shall be based on the unit price per linear foot as set forth in the Bid Schedule, per type or size of pipe. Said price shall include all costs necessary to complete the work including, but not limited to, all excavation (earth, rock, shale), dewatering, bedding, polyethylene encasement, placing, pipe to pipe connections, restraint measures, flushing, disinfection, pressure testing, backfilling, compacting, grading and removal of excess or unsuitable material, as required by the drawings and specifications.

- WATER LINE VALVES (ALL TYPES)

  - Final measurement will be based on each completed and installed item.

  - Payment for water line valves shall be based on the unit price per each as set forth in the Bid Schedule, per type or size of valve. Said price shall include all costs necessary to complete the work including, but not limited to, all excavation (earth, rock, shale), dewatering, bedding, polyethylene encasement, placing, pipe to pipe connections, restraint measures, flushing, disinfection, pressure testing, backfilling, compacting, grading and removal of excess or unsuitable material, as required by the drawings and specifications.

- WATER METER INSTALLATION

  - Final measurement will be based on each completed and installed item.

  - Payment for water meter installation shall be based on the unit price per each as set forth in the Bid Schedule.  Said price shall include all costs necessary to complete the work including, but not limited to, all excavation (earth, rock, shale), dewatering, bedding, polyethylene encasement, placing, pipe to pipe connections (including continuous copper pipe from meter to the main), restraint measures, flushing, disinfection, pressure testing, backfilling, compacting, grading and removal of excess or unsuitable material, as required by the drawings and specifications.  Owner will supply new meter, and contractor shall supply all the remaining parts to install the meter.

- WATER METER RELOCATION WITH REUSE OF EXISTING WATER METER

  - Final measurement will be based on each completed and installed item.

  - Payment for water meter relocation shall be based on the unit price per each as set forth in the Bid Schedule.  Said price shall include all costs necessary to complete the work including, but not limited to, all excavation (earth, rock, shale), dewatering, bedding, polyethylene encasement, placing, pipe to pipe connections (including continuous copper pipe from meter to the main), restraint measures, flushing, disinfection, pressure testing, backfilling, compacting, grading and removal of excess or unsuitable material, as required by the drawings and specifications.  Except for the actual water meter, contractor shall supply all parts to relocate the meter.  Contractor shall reuse the existing meter.  If meter cannot be reused, Owner will supply a new meter.

- WATER METER REMOVAL AND ABANDONMENT

  - Final measurement will be based on each completed and installed item.

  - Payment for water meter removal and abandonment shall be based on the unit price per each as set forth in the Bid Schedule.  Said price shall include all costs necessary to complete the work including, but not limited to, all excavation (earth, rock, shale), dewatering, corp removal, capping at the main, flushing, disinfection, pressure testing, backfilling, compacting, grading and removal of excess or unsuitable material, as required by the drawings and specifications.

- WIRELESS INTERCONNECT SYSTEMS (TRAFFIC SIGNALS)

  - Final measurement will be based on each completed and installed item.

  - Payment for wireless interconnect system shall be based on the unit price per each as set forth in the Bid Schedule.  Said price shall include all costs necessary to complete the work including, but not limited to, telemetry radios, antenna cable, connectors, surge arrestors, power supplies, interface cables, diagnostic pads and antennas, as required by the drawings and specifications.

END OF SECTION